PXD Components sensitive to radiation

Expected dose: there are no reliable simulations to date. Extrapolations from Belle are difficult, since important machine parameters change. Educated guess: 1 MRad/y

Assure that PXD components survive at least 10 Mrad.

Some active components will be in the DHH, about 30-50 cm from the PXD.

These will be some radiation in this area, how much is not known. It may be possible to shield the components.

|  | Process | Spec for Belle II |  | Tests |  |  | comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | x-Ray | MIP | NIEL |  |
| Component |  |  |  |  |  |  |  |
| DEPFET prototype (PXD4/5) | 200nm Oxide |  |  | 4 MRad |  | 3E12 n/cm² | Uthresh ~25V |
| DEPFET prototype (PXD6) | 200nm Oxide |  |  |  |  |  |  |
| DEPFET (final) | ~100nm Oxide | 10 MRad |  |  |  |  |  |
| DCD | UMC 180nm | 10 MRad |  | 7.5 MRad |  |  | Process rad hard |
| Switcher 3 | AMS 0.35 | 10 Mrad |  | 5.7Mrad |  |  | Process rad hard |
| Switcher 4 | AMS 0.35 HV | 10 MRad |  |  |  |  | To be tested |
| DHP | IBM 90nm | 10 MRad |  |  |  |  | Process rad hard |
| Optolink (DHH) | ? | ? |  |  |  |  |  |
| ASIC (DHH) Or FPGA ? | ? | ? |  |  |  |  |  |
| Regulator (DHH) | ? | ? |  |  |  |  |  |
| Test components |  |  |  |  |  |  |  |
| Diodes | 100nm Ox |  |  | 4 MRad |  |  | UFB ~2.4V |