Proposed Actions (December 8, 2009)

- EOS Geometry:  The proposal by Hans Krüger  is too long and the 1st layer needs to be split. Study alternative, more compact solutions (routing along the module, direct mounting of the Kapton with wire bond connections ?): Laci, Hans

The width of the Flex is then only 7mm and one needs two double layer or a multiple layer flex (Laci). Bond pitch for 80 connections is ok.

- DEPFET irradiations up to 10 Mrad in Karlsruhe

Scheduled for December (Andreas Ritter, Peter Müller)

- Switcher 4 radiation tests up to 10 Mrad (Heidelberg, MPI?)

Can be done in Karlsruhe, after (short) notice.

- Freeze module lengths (as soon as IR geometry is fixed) (TC)

- Produce thin dummy modules (Laci). Dimensions?

Preliminary dimensions (see above): 80 mm fro 1st layer, 122mm for 2nd. To be finished early next year. .

- Backside connection of DEPFETs: Laci, Rainer

Favourite option; punch through, To be tested

Tests started

- Joint for 2nd layer (and eventually 1st layer) half modules

Part of the dummy module production.

- Define max. length of inner layer module without split (->Rainer)

- Change (final) DHP design to work with up to 2000 lines (20µs readout cycle): Hans needs to know the exact number of lines to be read in order to design the PLL

- Footprint of the DHP test chip needs to be known to PXD6 designer by February/March 2010

Not really useful, since test chip can read only half the DCD channels. Better to use FPGA emulator and test DCD/DHP separately.

Can wait  some time. The PXD6 production will be split, and the second part can be prepared for the DHP.

- Finalize dimensions of DHH boxes and check if proposed space is sufficient (Ariane)

- Göttingen to check the cable layout by Hans-Günther (On Wiki).

- Propose common cool (and dry) volume for PXD and SVD. Create joint task force (ID Task force)

- Define module envelop for clearances (height of active and passive components and connectors, wire bonds etc).

Charly Ackermann can do the drawings, but needs dimensions of components. Linked to EOS layout.

- Check if cooling pipe insulation is needed for C3F8: no, should work without. Carlos Lacasta will discuss C3F8 cooling with ATLAS experts.

- Conceptual design of air cooling: MPI, Valencia, Karlsruhe

- What infrastructure (pipes…) is needed to supply 3-7 l/s of dry, cold air: Frank Simon to check wit engineer

Draft and distribute MoU on common fund (Christian), to be signed by X-mas

- Simulations should include background. Since background calculations are not yet available, study performance as function of occupancy (Christian)

- Order SOI wafers (March, 2010)

- Finalize parameters relevant for SOI order (sensor thickness) before March 2010. Need to decide in December

- Mock up of IR and inner PXD/SVD: IR task force

- PXD DAQ should be integrated in common DAQ in FY2010 (Giessen)

- Organize first meeting of ID task force (Ushiroda, Tsuboyama, Friedl, Moser, Shuji Tanaka, NN)

- Measure sensitivity of system to noise and pickup (MPI, Santiago)

- Grounding scheme (to avoid loops) (Santiago)

- Engineer module mounting without CVD. Direct contact to cooling block improves heat transfer. Possible problems: electrical contact, pressure on silicon. (MPI)

- Choose coolant; Water, CO2, C3F8 (MPI, Valencia, Karsruhe, KEK). Clean up TWIKI pages and interface to Belle II Twiki (?)

- Support for PXD on beam pipe (Shuji)

- Need information on NIEL and SEU

- Do we need hardware alignment monitor?

- Hans-Günther to buy a Phenix Audio System with echo suppression

- Check with IR group if ‘physics background (radiative Bhabha, pair production) should be simulated by MPI.

- Check assumption made for pair background estimate (1% occupancy)

- Christian to circulate TOC of PXD TDR