Belle II PXD EVO Meeting

23.3.2010

Present: H-G Moser, A. Ritter, C. Koffmane, J. Knopf, J. Ninkovic, A. Wassatsch, L. Andricek, M. Ritter, C. Kiesling,, Z. Dolezal, P. Fischer, Z. Drasal, B. Reisert, F. Simon, H. Krüger, H.-J. Simonis, T. Weiler , S. Fourletov, I Koroniv, M. Lemarenko, S. Rummel

Tuesday 23 March 2010; top. 10:00; Summary Services Workshop, March 8 (20') ( Slides ); Hans-Günther Moser.. 10:20; Report on visit of LHCb CO2 cooling system (20') ( Slides ); Christian Kiesling.. 10:40; DTC2 Measurements (10') ( Slides ); Jochen Knopf.. 10:50; PXD Session at B2GM (20') ( more information ); Hans-Günther Moser.. 11:10; Meeting on Background at B2GM (20'); NN.. 11:30; DAQ Session at B2GM (20'); NN.. 11:50; Common PXD/SVD/IR mechanics session at B2GM (20'); NN.. 12:10; AOB (20') ( list of actions )..

(http://indico.mppmu.mpg.de/indico/conferenceDisplay.py?confId=774)

- Report on Services, Power Supply meeting, March 8, 2010 (H.-G Moser)

The services chain should be as follows:

A Kapton link of 20cm-40cm length links the module to a patch panel located inside the shields, which need to be excavated. The length of the Kapton is limited by the transmission quality of the HF signals (clock, IO). The patch panel will have components for overvoltage protection and impedance matching to the ongoing twisted pair or coax cables which link to the DHH.

The DHH will be located in the docks, about 2m from the IP. Radiation level in this region to be understood. The DHH will have a FPGA for clock generation and control of the optolinks. Optolinks will be bi-directional: also clock and trigger will be optical. The need of local voltage regulators needs to be studied.

Grounding scheme needs to be designed from the beginning. It is recommended to have a local ground at the PXD (beam pipe?) and multipoint HF ground connection to avoid noise.

Responsibilities: Bonn: Kapton links (simulations and tests, prototypes have been ordered), Impedance matching in repeater box. Bonn needs help, especially for the LV part of the Kapton links and patch panels (Ramon Llull?)

DHH to be designed, prototyped and tested by TU Muncih (with help of Ramon Llull for optocomponents).

LMU Munich and Cracow will do tests on the PS system (need of regulation)

Santiago de Compostella is looking into grounding and shielding issues.

Q: tapered Kapton links to improve resisitivit. A (H.Krüger): Makes HF transmission worse.

- Report on visit of LHCb CO2 cooling system (Christian Kiesling)

March 11 Christian Kiesling, Hans-Jürgen Simons, Imanuel Gfall, Thomas Weiler, Carlos Marinas and Hans-Günther Moser visited CERN to see the LHCb CO2 cooling system and the test systems by Hans Postema.

The LHCb system has been working for two years. The system is fully engineered and can be easily copied without much R&D need. Test systems can be rather simple, just CO2 form the bottle without need of recirculation.

The only disadvantage is the need of cold supply and return lines, which, however, is manageable. An alternative system using C3F8 which could be designed with warm lines is much more complicated and support is limited.

The conclusion was that the CO2 cooling is the most attractive and we can expect much support (NIKHEF).

Recommendation: CO2 baseline, Water cooling as backup, C3F8 will be abandoned. Open cooling systems (bottles) will be used by Valencia and Karlsruhe to test local properties (heat conductivity, temperature of cooling blocks, pressure drops and mass flow needed). The recirculating system should be engineered at KEK.

- Status of DCT (Jochen Knopf)

The DCT is a mini-ASiC containing DCD-B like logic blocjs (digital parts of the DCD-B) for testing. The tests have started and first results were presented. The chip works wll up to 600 MHz (Target frequency 400 MHz) with supply voltages below 1.6V (nominal 1.8V). A spike at 450 MHz needs to be understood.  So the digital part is faster than needed, however, the ADC of the DCDB might be the limiting device.

Everybody congratulated Jochen (and Mannheim) for the excellent work.

- PXD Session at B2GM

The proposed agenda for the PXD session in the next B2GM is:

Agenda PXD Meeting, KEK, April 1st, 9:00-13:00

9:00	SOI Status		Yasuo Arai

9:20 	DEPFET Overview 	Christian Kiesling

9:40	PXD6 status 		Jelena Ninkovic

10:00 	Digitizer 		Zbynek Drasal

10:20	Module Design		Laci Andricek

10:40	Mechanics & glue tests	Martin Ritter

11:00	Coffee

11:20  	End Flange Test	Hans-Jürgen Simonis*)

11:40	Thermal Simulations	Carlos Marinas

12:00 	Performance Studies I	Burkhard Reisert

12:20	Performance Studies II	Thomas Müller

12:40	DCD Tests		Sergey Fourletov

Christian should cover (amongs other things) the ASIC submissions and ASIC tests (irradiations)

Jelena’s talk should report on the recent DEPFET irradiations.

Hans-Jürgern should shortly mention the CO2 system but this topic should mainly be covered in the common IP/PXD/SVD session

Presently two talks are scheduled on Performance studies, but Burkhard and Thomas should get into contact and see if this can be merged.

- Background Session at B2GM

The pair background studies done at MPI seem to converge. The (high) numbers from SuperB still need to be understood. We still need more information on machine background. Since Masako is now joining the KEK machine group it is not clear who will work on this.

- DAQ Session at B2GM

Plenty of progress can be reported: the funding problem of Giessen seems to be solved. They will have a PhD student dedicated to Belle II and can submit a new application to BMBF. As backup of the ATCA system a PC based system is discussed. Deadline for a decision is end of 2010.

Christian will report on the data reduction algorithms (work by ??)

- Common IP/PXD/SVD session at B2GM

This should be a discussion session. Topics to be discussed and presented are of course CO2 cooling. Tsuboyama-san will discuss the agendain the SVD EVO meeting, March 24. Hans-Jürgen and Carlos will join this meeting.

- AOB

We went through a part of the action list which has been updated (see below).

The next  EVO meeting will be April 13.

Proposed Actions:

- (A1.1 from A1 and A13) EOS Geometry:  Since the first layer will be split as well, the constraints on the length of the EOS are now more relaxed and can be revised. Define module envelop for clearances (height of active and passive components and connectors, wire bonds etc).

Charly Ackermann can do the drawings, but needs dimensions of components

2	.  (A3) Freeze module lengths (as soon as IR geometry is fixed) (TC)

3.   (A4) Backside connection of DEPFETs: Laci, Rainer

Favourite option: punch through. Tests started by Christian Koffmane, looks promising. Backup solution in work

- (A8.2 from A8.1) Revise functionality of DHH assuming a patch panel can be used to have it farer from the IP. Finalize dimensions and location of these boxes: DHH will be in the Docks, 2m from PXD. Need of local regulation in DHH to be checked (LMU, Kracow). DHH Design by TU

- (A9) Göttingen to check the cable layout by Hans-Günther (On Wiki).

- (A10) Conceptual design of air cooling: MPI, Valencia, Karlsruhe

- (A11.1 from A10, A11 and A16) Design common cool (and dry) volume for PXD and SVD air cooling system. Define pipe dimensions (Carlos Marinas and I. Fall from Vienna)

- (A17.1 from A17) Simulations should include background. Since background calculations are not yet available, study performance as function of occupancy (Christian). In work. Various backgrounds can be mixed in MC events. Results beginning of March. Procedure is set up but the study still needs to be done (Kolja: for B2GM)

- (A19) Finalize parameters relevant for SOI order (sensor thickness) before March 2010. Need to decide in December: Wait for A18

- (A20) Mock up of IR and inner PXD/SVD: IR task force: First space problem has to be solved and drawings need to be prepared

- (A21) PXD DAQ should be integrated in common DAQ in FY2010 (Giessen). In work. Need to decide whether ATCA or eventbuilder farm will be used.

- (A22.1) Organize first meeting of ID task force (Ushiroda, Tsuboyama, Friedl, Moser, Shuji Tanaka, NN): April 1st at B2GM?

- (A23) Measure sensitivity of system to noise and pickup. Stefan Rummel will work on pickup measurements

- (A24.1) Grounding scheme (to avoid loops) (Santiago). First recommendations available (Santiago). Will be discussed in special PS & Services meeting early March. Discussed (F. Arteche). Santiago continues to work on this (23.3.2010)

- (A26) Support for PXD on beam pipe (Shuji). Shuji will prepare schematic drawings and send them to Charlie.

- (A27) Need information on NIEL and SEU. NIEL damage might be severe. Background estimate needed and irradiations with electrons are needed.

Special run with separated beams planned.

- (A28) Irradiate DEPFETs with 10MeV electrons at ELSA, Bonn (Ritter, Müller). Scheduled for April

- (A33) Feedback to Shuji on PXD assembly, installation and service routing

- (A35) Check legal constraints for high pressure pipes in Japan (Shuji): Shuji will find out and post it on our TWiki.

- (A36) Resolution studies should also look at the digitisation noise of the ADC.

- (A37.1) Continue with pair background studies. Look in ‘Traps’ program. There exists another program used by Masako – check!

Understand superb numbers. Who will replace Masako? (23.3.2010)

- (A40) Do some voltages need current sinks?

27. (A41.1) Valencia, will concentrate on open system without recirculation and     eventually upgrade to an recirculation system (23.3.2010).

28.(A42) Frank Simon to find out about rad hard Gigabit optical links (2.3.2010)

29.(A43) Get information on the expected radiation level at the locations of the opt. links (2.3.2010)

30.(A44) Peter Fischer to check effect of electromigration in switcher B (2.3.2010)

31.(A45) Peter Fischer to write a list of supply voltages and ratings of switcher B (2.3.2010)

33.(A47) Check whether the scheduled background runs at Belle can be done with a reduced B-field (2.3.2010)

34.(A48.1) Study effect of beam pipe material on impact parameter resolution (2.3.2010). E.g. use Babar (4 µm Au) as benchmark (23.3.2010)

36. (A50) Select decoupling caps for the switcher and calculate the contribution to the X0.

34 (A51)Test HF properties of Kapton + impedance match (Bonn) (8.3.2010)

- (A52) Design prototype patch panel (Bonn?) (8.3.2010)

- (A53) Test need of regulation and overvoltage protection (LMU, Kracow) (8.3.2010)

- (A54) Ground connection to beam pipe (KEK) (8.3.2010)

- (A55) Need of shielding between SVD/PXD (discuss in B2GM, March 2010) (8.3.2010)

- (A56) Irradiation tests of SODA/DHH components (TU) (8.3.2010)

- (A57) Grounding studies and PS prototype (Santiago de Compostella) (8.3.2010)

- (A58) Qualify SOI wafers from LETI and ICEMOS (23.3.2010)

- (A59) Visit NIKHEF for CO2 cooling (23.3.2010)